STM32F407ZGT6-6-TFTLCD-Basic

# Circuit Connection

LCD_BL连在PB15上, 是GPIO;

上图是原子F4主板上的接口图, 与之对应的LCD上的连接是:

# LCD Controller Datasheet

我的板子的LCD是4.3英寸的, ID是5510. 它的Controller是NT35510.

NT3550支持很多种不同的Interface, 我这里用的是8-/16-/24-bits 80-series MPU interface.

## 2.1	MPU Interface

The 8080-series bi-directional interface can be used for communication between the micro controller and LCD driver chip. Interface bus width can be selected with IM3,IM2, IM1 and IM0.

### WRITE CYCLE SEQUENCE

The write cycle means that the host writes information (command or/and data) to the display via the interface. Each write cycle (WRX high-low-high sequence) consists of 3 control (D/CX, RDX, WRX) and data signals (D[23:0]). D/CX bit is a control signal, which tells if the data is a command or a data. The data signals are the command if the control signal is low (=’0’) and vice versa it is data (=’1’).

### READ CYCLE SEQUENCE

The read cycle (RDX high-low-high sequence) means that the host reads information from display via interface. The display sends data (D[17:0]) to the host when there is a falling edge of RDX and the host reads data when there is a rising edge of RDX.

### 16-BIT PARALLEL INTERFACE FOR DATA RAM WRITE

### 16-BIT PARALLEL INTERFACE FOR DATA RAM READ

## 2.9 Frame Memory

The NT35510 has an integrated 480 x 864 x 24-bit graphic type static RAM. This 9,953,280 bit memory allows to store on-chip a 480 x RGB x 864, 480 x RGB x 854, 480 x RGB x 800, 480 x RGB x 720 and 480 x RGB x 640 image with an 24-bit resolution (16.7M-color).

Before writing to the RAM a window must be defined. The window is programmable via the command registers XS, YS designating the start address and XE, YE designating the end address. For example, the whole display contents will be written when CGM[7:0]=”50h”, if the window is defined by the following values: XS=0 (0h) YS=0 (0h) and XE=479 (1DFh), YE=799 (31Fh).

### 2.9.3	Interface to Memory Write Direction

## 7.6	Parallel Interface Characteristics (80-Series MCU)

The figures above are the timing of NT35510. In our code example, we are using FSMC Mode A for Read and Write, and its diagrams are:

Here, ADDSET is a 4-bit register portion while DATAST is a 8-bit register. Considering we are using 168MHz HCLK, and its CLK cycle is around 6ns. Our code example sets the two time period of Write to 18ns, while for Read, setting the ADDSET to 96ns, and DATAST to 360ns. However, according to test, DATAST can be set to as low as 30ns.

# FSMC Read/Write to LCD Controller

typedef struct

{

uint16_t LCD_REG;

uint16_t LCD_RAM;

} LCD_TypeDef;

#define LCD_BASE        ((uint32_t)(0x6C000000 | 0x0000007E))

#define LCD             ((LCD_TypeDef *) LCD_BASE)

This code example uses FSMC_Bank1_NORSRAM4. According to Reference Manual documents：

Bank1 – SubBank 4 starts with 0x6C000000.

FSMC uses address signals HADDR[27:0] to address in Bank 1 256M space. HADDR[27:26] determines which sub-bank, and HADDR[25:1] is assigned to FSMC_ADDR[24:0] when data width is 16-Bit.

0x7E is 0111 1110, so LCD_REG is @ [0111 111]0, and LCD_RAM is @ [1000 000]1. Pay attention to the address in [], there is transition of A6 from 0 (Command) to 1 (Data and parameters).

# LCD Init

Here we will talk about several registers.

LCD resolution setting is WIDTH = 480 (X-Axis), HEIGHT = 800 (Y-Axis).

## Column Address Set (2A00h~2A03h)

## Row Address Set (2B00h~2B03h)

## Memory Write (2C00h)

## Memory Read (2E00h)

Memory Write and Memory Read is working with the coordination of X-Y axis.

## Interface Pixel Format (3A00h)

## MADCTL: Memory Data Access Control (3600h)

This register is set to 0x0000 in our code example.

- MV-MX-MY is used to set the orientation between MPU and GRAM.

- ML-MH is used to set the refresh orientation.